SECTION 32 31 13

ARCHITECTURAL HEAVY DUTY ALUMINUM WITH ALUMINUM WIRE INFILL

The specifier will need to edit this product specification for a specific project to reflect the options and applications being used.  This guide section has been written to show items to be determined reflected in italic red.  PalmSHIELD does not warrant that these specifications are complete and applicable to all applications.  It is the specifier’s responsibility to assure that use of PalmSHIELD is an appropriate fit for the site and conditions.

PART 1 – GENERAL

  - DESCRIPTION

This section describes the following fence system:

Gate panels fabricated with extruded aluminum planks and structural aluminum profiles including extruded aluminum framework and aluminum infill materials. Gate panels shall be furnished and installed as shown on the plans and specified herein, overall height and opening width of gates shall be as shown on the plans

  - REQUIREMENTS

- Furnish materials, labor, expertise and equipment necessary to complete all work specified in this section and as shown on the drawings.

- Structural Performance:  Provide product and installation capable of withstanding the effects of gravity loads and the following loads and stresses within limits and under conditions indicated.

    - Uniform pressure of 30 lbf/sq. ft. acting inward or outward.

    - Thermal Movements resulting from a temperature change (range) of 120 degrees Fahrenheit ambient  and 180 degrees Fahrenheit material surfaces.

  - SUBMITTALS

- Shop drawings and manufacturer's literature:  Provide specifications and construction detail drawings to substantiate quality of materials and provide details of fabrication and installation.

- Submittals shall be in accordance with standard construction practices to include complete detailed layout of all gate panels and gate posts.  Submittals shall include plan layout, elevations and section views of gates.

- Certificate:  manufacturer's certification that materials meet specification requirements.

  - REFERENCES

- ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet 				and Plate.

- ASTM B 221 – Standard Specification for Aluminum and Aluminum-Ally 					Extruded Bars, Rods, Wire, Profiles and Tubes.

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test.

- ASTM D2794 – Standard Test Method for Resistance of Organic Coatings to the 				Effects of Rapid Deformation.

- ASTM B117 – Standard Practice for Operating Salt Spray Apparatus.

- ASTM D822 – Standard Practice for Filtered Open-Flame Carbon-Arc Exposures of 			Paint and Related Coatings.

- AWS D1.2 Structural Welding Code – Aluminum.

  - QUALITY ASSURANCE

- Installation of fence and materials shall conform to the requirements of the fence manufacturer.

- The fence shall be warranted from any defects in materials and workmanship for a period as specified in the relevant section of the contract documents.

PART 2 – PRODUCTS

  - MATERIALS

- Gate panels:

  - Approved architectural gate panels and gate posts, Manufacturer:

    - PalmSHIELD Manufacturing. Maximus gate design

    - Heavy duty swing gate design

- Material Descriptions:

  - Extruded Aluminum: ASTM B 221, Alloy 6063 – Temper T-6.

  - Sheet Aluminum: ASTM B211, Alloy 6063 – Temper T6.

  - Powder Coating Material Hardness: ASTM 3363 2H.

  - Gate Panel Infill Description:

    - Gate Height: State here

    - Individual Panel Width: State here.  Widths over 12’, verify with manufacturer.

    - Gate panels consisting of wire mesh fabricated with extruded aluminum framing structural profiles and supported by extruded aluminum or steel fence posts.

    - Gate panel infill framework to be fully welded picture frame 3” x 3” x ¼” 6061 T6 aluminum structural angle.

    - Square mesh security fence:  Approved square mesh fence system, Manufacturer.  “PalmSHIELD”

    - Material Descriptions:

    - Square opening (4”, 2”, 1 ½”, 1”)

    - Wire size (.25”, .120”)

    - Lockcrimp weave

    - Most common wire mesh:  Square, Aluminum, Alloy 1350-H18, Woven - Lock Crimp Weave, 2.0000" x 2.0000" Opening (Square), 0.250" Thick (2-3/4 Gauge) Wire Diameter, 79% Open Area

  - Gate Frame Description:

    - Gate shall have welded frame fabricated from extruded aluminum tubing with aluminum panels to match fencing material. Frame configurations shall be as indicated on the contract drawings.

    - Welded frame, 2” x 4” extruded tubular 6063 aluminum to be welded to panel framework.

  - Gate Posts:

    - Gate panel post selection as shown on the plans and per manufacturer’s selection.

    - Gate posts may be T-6063 aluminum or A-500 tubular steel as per manufacturer.

    - Gate posts will be capped.

  - Anchor Bolts: Anchor bolts shall be (state means of anchoring posts to adjoining surface) and adequate to support loads based on screening height, exposures and loading.

  - Gates Hardware:

    - All fittings and accessories shall be stainless steel and sized as specified by the fence manufacturer.

    - Gate hinges to be Gorilla barrel hinge with ¾” rod, ball bearing, and grease zert.  Hinge plate to be ½” thick plates offset to create a 5/8” gap.  Standard hardware as required by the gate manufacturer for complete functional operation.  Hinges to be bolted to gate frame and field welded to steel gate posts.

    - Gate look to be Locinox FortyLock mortise lock.  Lock to be industrial and polished 100% stainless steel locksets include a keyable cylinder and auto-latching mechanism.  Lock to be internal lock with exterior grab handles.  Lock may be keyed and rekeyed.  Lock is accessible from both sides of gate.

    - Drop rods to be 1” schedule 40 pipe and through bolted to gate frame.

    - Hardware: Size and type as determined by the manufacturer. Provide 	two hinges per leaf.

    - Gate posts shall be as determined by manufacture.  Gate posts to be specified to support gates.

6.	Factory Finish: Gates shall receive polyester powder coating.

a.	Polyester powder coating: Electrostatically applied colored polyester 			powder coating heat cured to chemically bond finish to metal 			substrate.

b.	Color shall be as selected by Owner.

c.	Minimum hardness measured in accordance with ASTM D3363 2H.

d.	Direct impact resistance tested in accordance with ASTM D2794. 			Withstand 160 inch-pounds.

e.	Salt spray resistance tested in accordance with ASTM B117: No 			undercutting, rusting, or blistering after 500 hours in 5 percent salt 			spray at 95° F and 95% relative humidity after 1,000 hours, less than 			3/16 inches undercutting.

f.	Weatherability tested in accordance with ASTM D822: No film 			failure and 88 percent gloss retention after 1 year exposure in South 			Florida with test panels tilted 45°.

PART 3 – EXECUTION

  - INSPECTION

  - Verify that final grading in gate location is completed and without irregularities which will interfere with gate installation.  Variations in height, slopes, stairs steeping shall be shown on contract drawings and on submittal drawings.

  - Field verify all dimensions and layout prior to commencing installation.

  - Do not commence work until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

- Install gate in accordance with manufacturer’s installation instructions.

- Install gate plumb and level. Posts are plated and mounted to top of surface or as shown on the plans.

- Do not install bent, bowed or otherwise damaged gate panels or posts. Remove damaged components from site and replace.

- Install gates and adjust hardware for smooth operation